NO-SPILL® AUTOSTOP Function

We sometimes have customers that write or call and say the autostop doesn’t work for them.  The following may help you understand more about the autostop function:

How it works:

There is a central straw in the nozzle tip that carries vent air into the gas can.  This vent air allows the patented No-Spill nozzle to flow fuel without gurgling.  When the fuel gets to the nozzle tip, this cuts off the vent air and the fuel flow stops.

CARB test tank

The NO-SPILL autostop works within the CARB (California Air Resources Board) guidelines.  CARB has designed a standard test tank to be used for such purposes.  It is by this standard which we and others judge our autostop function.  It will stop additional fuel from flowing once it reaches our spout in the CARB test tank and in almost every other tank but the most difficult ones.

Tips to make the Autostop work best:

- Hold the nozzle vertical to the target tank

- Partially rest the spout’s catch on the target tank’s neck or put the nozzle as far as possible into the target tank

- Press the button and hold it down firmly.

- When the fuel stops flowing, let the button go then lift the nozzle up.

Problem?  The autostop won't work if:

- The seal between the nozzle assembly and can neck leaks air.

- The target tank has a narrow neck and the fuel rises too fast for the autostop to work.

- The button isn't depressed fully.

Remedies:

Be sure the seal between the nozzle and gas can is a good one.

Put the end of the spout further into the target tank.

Press the button all the way down and hold it.

If it still won't autostop, just release the button when you want the fuel to stop.  The target tank’s geometry must make the gas rise too quickly to work in this instance.